Automatic Garden Plant Watering System Using STM32F407G-DISC1

Below are the peripherals you will use and a clean roadmap you can follow step-by-step to finish the project.

✅ 1. Peripherals Required (with purpose)

🔸 1. GPIO (Digital Input/Output)

Used for:

- Driving relay/MOSFET for water pump

- Reading float switch (tank empty)

- Reading manual push button (override ON/OFF)

- Reading rain sensor (digital type)

✔ Essential peripheral

🔸 2. ADC (Analog-to-Digital Converter)

Used for:

- Reading soil moisture sensor (capacitive analog type recommended)

- Reading water level sensor (optional)

- Reading potentiometer for threshold setting (optional)

✔ ADC1 or ADC3 on pins like PA0, PA1, PB0, PC1 etc.

🔸 3. TIMERS (General Purpose Timers)

Use two types:

A. Delay timer

- For periodic moisture checking (1 sec, 5 sec, etc.)

- For pump run time measurement

B. PWM timer (optional)

To control:

- Pump speed (if DC pump supports PWM)

- Valve position (servo)

Examples:

- TIM2 (32-bit, perfect for precise delays)

- TIM3/TIM4 for PWM (DISC1 LEDs are on TIM4)

🔸 4. UART (Serial communication)

Used for:

- Printing moisture value

- Debugging

- Changing threshold via terminal

- Connecting Bluetooth module (HC-05)

UART2 is easiest (connected to ST-LINK Virtual COM Port).

🔸 5. RTC (Real-Time Clock)

Used when you need scheduled watering:

- Water at 7 AM daily

- Water every 48 hours

- Skip watering at night

STM32F407 has built-in RTC module.
(Needs a coin cell battery).

🔸 6. I2C (Optional but recommended)

Used for:

- LCD 16×2 I2C

- OLED display

- External RTC like DS3231

🔸 7. EXTI (Interrupts)

Used for:

- Float switch emergency stop

- Rain detector interrupt

- Manual button press

- Watchdog timer triggers

🔸 8. Watchdog Timer (Recommended for reliability)

Used to restart system if stuck.
Useful for long-running outdoor systems.

⭐ Summary of Peripherals Used

| Peripheral | Purpose |
| --- | --- |
| GPIO | Pump, rain sensor, float switch, buttons |
| ADC | Soil moisture, water level |
| Timer | Periodic tasks, pump timing, PWM |
| PWM | Pump speed / valve control (optional) |
| UART | Serial debug / Bluetooth |
| I²C | LCD/OLED / External RTC (optional) |
| RTC | Scheduled watering |
| EXTI | Emergency stop, buttons |
| Watchdog | System reliability |

🌱 2. Roadmap for the Project (Step-by-Step)

⭐ STEP 1: Basic Setup (Day 1)

- Install STM32CubeIDE

- Test LED blinking (verify code works)

- Enable UART for debugging (printf to serial monitor)

📌 Outcome: Board is ready for development.

⭐ STEP 2: Soil Moisture Sensing (ADC)

- Connect capacitive soil moisture sensor to ADC pin (e.g., PA0)

- Read ADC value

- Convert to percentage moisture

- Apply simple filtering (average of 10 samples)

📌 Outcome: System can measure soil moisture accurately.

⭐ STEP 3: Drive Water Pump (GPIO + MOSFET/Relay)

- Build pump driver using IRLZ44N MOSFET or relay module

- Add flyback diode (for DC pump)

- Control ON/OFF using GPIO pin

Add float switch (tank empty safety)

- If float switch = LOW → stop pump.

📌 Outcome: STM32 can control pump safely.

⭐ STEP 4: Automatic Watering Logic

Implement basic hysteresis:

if (moisture < 40%) → pump ON

if (moisture > 60%) → pump OFF

Add safety:

- Float switch

- Rain sensor

- Minimum ON time (e.g., 5 sec)

- Minimum OFF time (e.g., 60 sec)

📌 Outcome: System waters automatically based on moisture.

⭐ STEP 5: Add Timers (for periodic checks)

- Check moisture every 2 seconds

- Limit pump running time

- Use TIM2 or SysTick

📌 Outcome: Smooth timing-based operation.

⭐ STEP 6: Add Display / User Interface (Optional)

Using I2C:

- Show soil moisture

- Pump ON/OFF status

- Tank empty alarm

- Set threshold using buttons

📌 Outcome: User can see real-time status.

⭐ STEP 7: Add RTC for Scheduled Watering

- Use STM32 RTC or DS3231

- Water only in morning/evening

- Add "skip watering if raining"

📌 Outcome: Intelligent watering system.

⭐ STEP 8: Add Bluetooth / Wi-Fi (Optional)

- Control via phone

- Send moisture data

- Receive pump ON/OFF commands

📌 Outcome: Smart, modern system.

⭐ STEP 9: Long-Term Reliability

- Use watchdog timer

- Use fuses and TVS protection

- Add IP56 waterproof enclosure

📌 Outcome: Can run in outdoor conditions 24×7.

🌟 Final System Features (What You Will Achieve)

Your final project will:

✔ Measure soil moisture
✔ Water plants automatically
✔ Protect pump (tank empty detection)
✔ Avoid watering when raining
✔ Run on schedule (morning/evening)
✔ Show real-time status
✔ Handle emergencies
✔ Run months without failure